TU Transaction Control registers

Overview:

This document describes how to control the TU to generate /RLTDs (Transaction requests).

General Operation:

The general operation is to set the pulse duration for each signal, the number of major and minor cycles to run, the time between each minor cycle and the time between each major cycle.  The desired steering words for Alpha and Beta are also set.  The requested transactions are started by toggling a bit in the main control register from 0 to 1.

Note that when using /RLCP or /RLSC command bits in the main control register must be set in addition to their durations.

There are sub-setting for performing a “stress test” that allow setting an increment value for Alpha and Beta steering words in addition to a value that determines how many times to increment before returning to the starting value and begin incrementing again.  The increment occurs for each new transaction regardless of major or minor cycle.

Time Settings:

All “time based” settings are defined in clock counts.  The clock period is 10ns.  The following equation can be used to determine clock count for a specific time.

Counts=Desired Times10ns

Registers:

There are several registers that define the waveforms the TU can generate.  The TU generates a total of 23 output signals

- /RLTD

- /RLCP

- /RLSC

- Alpha_Steering_Word[9..0]

- Beta_Steering_Word[9..0]

All signals have a duration setting to define how long they are “active” once a transaction starts (the alpha and beta steering word signals all have the same duration specified by a single register).

Two registers define how many “burst” transactions (/RLTDs) should occur and what the time between each transaction should be.

Two more register define how many times the “burst” should occur and the time between each “burst”.

Three additional registers can be used to  support a “stress test”.  Two of these registers define a fixed increment value of the Alpha and Beta steering word values.  The start value is the currently programmed Alpha/Beta steering words in their respective registers. The third register defines how many times to increment the steering word before starting back at the original steering word and incrementing again.  The increment occurs at each transaction (/RLTD).  Set this register to 0 or set the increment values to 0 when not running the stress test.

| Name | Offset Addr | Description |
| --- | --- | --- |
| ARM KSine Alpha | 0x000C | Alpha Steering Word |
| ARM KSine Beta | 0x0010 | Beta Steering Word |
| /RLTD Duration | 0x0038 | Duration of /RLTD (typically 1us) |
| /RLCP Duration | 0x003C | Duration of /RLCP |
| /RLSC Duration | 0x0040 | Duration of /RLSC |
| SW Duration | 0x0044 | Duration of Alpha and Beta Steering words (typ. 3us) |
| /RLTD Period | 0x0048 | Defines spacing between each /RLTD |
| /RLTD Runs | 0x004C | Defines |
| Num Increments | 0x0054 | # of /RLTD to increment Alpha/Beta before starting over |
| Alpha Increment | 0x0058 | Value to increment the starting value by each /RLTD |
| Beta Increment | 0x005C | Value to increment the starting value by each /RLTD |
| Cycle Time | 0x0060 | Time between running Number of /RTLDS at the /RTLD period |
| Num Cycles | 0x0064 | Number of times to run the number of /RTLDS at the /RTLD period |
| Main Control | 0x0008 | Toggle Bit2 from zero to 1 to start the specified transactions (note bits in the register start at Bit0) |

Example 1:

Address          Data

0x000C : 0x00000001

0x0010 : 0x00000002

0x0038 : 0x00000064

0x003C : 0x00000000

0x0040 : 0x00000000

0x0044 : 0x0000012C

0x0048 : 0x000007D0

0x004C : 0x00000003

0x0054 : 0x00000003

0x0058 : 0x00000001

0x005C : 0x00000004

0x0060 : 0x00000FA0

0x0064 : 0x00000003